QCE EARTH & ENVIRONMENTAL SCIENCE · UNIT 3 TOPIC 1: USE OF NON-RENEWABLE EARTH RESOURCES

# Coal seam gas: practice data test.

Name: ______________________________     Class: ____________     Date: ____________

Perusal: 2 minutes. Working time: 25 minutes. You may use a calculator. Datasets A and C are model data; dataset B is real. Answer in the spaces on pages 3 and 4, and show your working for calculations.

DATASET A   Well 4 and Block D	Model data

| Well 4: density log Depth below the surface (m) down the side; density (g/cm³) across. Each reading covers 0.5 m. | Well 4 is in Block D, which covers 1.5 km². The coal in Block D has a density of 1.45 g/cm³. Three samples of the coal from Well 4 were tested for their gas content. Gas content of the coal samples Sample; Gas content (m³/t). 1; 3.8. 2; 4.6. 3; 5.4. Coal with little ash is below 1.75 g/cm³, and coal with a lot of ash below 2.0 g/cm³ (Lange, 2003). The dashed lines on the log mark 1.75 and 2.0. |
| --- | --- |

DATASET B   Water bores predicted to be affected	Real data

Long-term affected area: bores predicted to pass the trigger threshold at any time

| Formation, in the order OGIA lists them | Bores |
| --- | --- |
| Alluvium (Main Range Volcanics) | 1 |
| Westbourne Formation | 9 |
| Gubberamunda Sandstone | 2 |
| Springbok Sandstone | 103 |
| Walloon Coal Measures | 321 |
| Hutton Sandstone | 8 |
| Bandanna Formation | 1 |
| Existing bores | 445 |
| Bores with make good agreements, decommissioned, or of uncertain status | 302 |
| All long-term affected bores | 747 |

Immediately affected area: bores predicted to pass the trigger threshold in 2026 to 2028

| Formation | Bores |
| --- | --- |
| Springbok Sandstone | 20 |
| Walloon Coal Measures | 56 |
| All immediately affected bores | 76 |

Most of the gas comes from the Walloon Coal Measures. The Springbok Sandstone lies above them, and the Hutton Sandstone below. A bore’s trigger threshold is a fall in water level of 5 m in a consolidated aquifer, or 2 m in an unconsolidated aquifer.

Source: OGIA, Underground water impact report 2025 for the Surat Cumulative Management Area, consultation draft, Tables 11-1 and 11-4. The parts of the Springbok Sandstone and the Walloon Coal Measures are added together.

DATASET C   Two monitoring bores	Model data

| Bore X, sandstone Drawdown since 2019 (m), with the trigger threshold of 5 m (dashed). | Bore Y, river alluvium Drawdown since 2019 (m), with the trigger threshold of 2 m (dashed). |
| --- | --- |

| Drawdown (m) | 2019 | 2020 | 2021 | 2022 | 2023 | 2024 | 2025 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bore X | 0.0 | 0.6 | 1.3 | 1.9 | 2.4 | 3.1 | 3.6 |
| Bore Y | 0.0 | 0.1 | 0.1 | 0.2 | 0.2 | 0.3 | 0.3 |

Bore X draws water from sandstone, a consolidated aquifer. Bore Y draws water from river alluvium, an unconsolidated aquifer.

Questions. The tag beside each question says what kind it is and which dataset it uses.

1	WORK OUT · DATASET A  What is the net thickness of coal with little ash in Well 4?

2	WORK OUT · DATASET A  Using the middle gas content, calculate the gas in place in Block D. Show your working.

3	WORK OUT · DATASET C  Calculate the average rate of drawdown in Bore X from 2019 to 2025.

4	TRENDS AND LIMITS · DATASET B  Identify the pattern in which formations hold the existing long-term affected bores.

5	TRENDS AND LIMITS · DATASET C  Compare the trends in Bores X and Y.

6	TRENDS AND LIMITS · DATASET A  Identify two sources of uncertainty in your answer to question 2.

7	CONCLUDE · DATASET C  Predict when Bore X will pass its trigger threshold, and state the assumption you made.

8	CONCLUDE · DATASET C  In 2025, should Bore Y have a bore assessment? Justify your answer.

9	CONCLUDE · DATASET B  Infer why most of the affected bores draw water from the Walloon Coal Measures.

10	CONCLUDE · DATASET A  Block D will only make a profit if it holds at least 70 million m³ of gas. Using your answers to questions 2 and 6, justify whether the company should develop Block D now. Use up to 150 words.